# Indicative Cost & Bill of Materials (BOM) — OAG Delta · QUOT PSA

Date: 07/06/2026 · Currency: NGN (USD shown for import‑priced hardware @ ~₦1,550/US$)

⚠️ Indicative only. Hardware/cloud figures are planning estimates and require formal vendor quotes. Lines marked [Vendor commercial] (software licence, implementation, migration, training, support) are set in the vendor’s commercial proposal and are not estimated here. Final sizing depends on confirmed user counts, data volumes, and facility readiness.

## A. One‑Time — Hardware & Facility (Scenario ① On‑Premise Primary)

| # | Item | Spec (summary) | Indicative NGN | USD ref |
| --- | --- | --- | --- | --- |
| 1 | Application/DB server | Dell R760 / HPE DL380 Gen11 · Xeon Gold (high‑clock) · 128 GB ECC DDR5 · NVMe RAID‑10 (DB) + RAID‑1 (OS) · redundant PSU · dual 10 GbE | ₦9,000,000 – 18,000,000 | $6k–12k |
| 2 | Online UPS (double‑conversion) | 6 kVA + extended batteries, 30–60 min runtime | ₦1,500,000 – 3,500,000 | — |
| 3 | Standby generator + ATS (if not already available) | ~15–20 kVA, automatic change‑over | ₦3,500,000 – 9,000,000 | — |
| 4 | Backup NAS | RAID, ≥8–16 TB usable (local nightly backups) | ₦1,000,000 – 2,500,000 | — |
| 5 | Firewall / UTM | FortiGate/Sophos class + 1‑yr security licence | ₦1,500,000 – 4,000,000 | — |
| 6 | Network & server‑room provisioning | Managed switch, rack, cabling, cooling (AC), access control | ₦1,500,000 – 4,000,000 | — |
|  | Subtotal — Hardware & Facility |  | ₦18,000,000 – 41,000,000 |  |

If OAG already has generator/cooling/rack, deduct items 3 and part of 6.

## B. Recurring — Infrastructure (per scenario, annual)

Scenario ① — On‑prem primary + Nigerian cloud DR (warm standby + backups):

| Item | Spec | Indicative NGN / yr |
| --- | --- | --- |
| Cloud DR (Nigerian Tier‑III) | Warm standby (smaller than primary) + object storage + encrypted backups | ₦3,000,000 – 7,200,000 |
| Subtotal ① (annual) |  | ₦3,000,000 – 7,200,000 |

Scenario ② — Nigerian sovereign‑cloud primary + on‑prem local backup:

| Item | Spec | Indicative NGN / yr |
| --- | --- | --- |
| Cloud production | App (8–16 vCPU) + PostgreSQL (8 vCPU/32–64 GB/NVMe) + Redis + object storage + snapshots, ≥99.9% SLA | ₦7,200,000 – 18,000,000 |
| Dual ISP at OAG (resilience) | Two independent links | ₦1,800,000 – 4,800,000 |
| On‑prem local backup (one‑time) | NAS + small UPS (one‑time, see A4) | (one‑time) |
| Subtotal ② (annual) |  | ₦9,000,000 – 22,800,000 |

## C. Software, Implementation & Services (vendor commercial — insert figures)

| Item | Basis | Amount |
| --- | --- | --- |
| QUOT PSA platform licence | Per named user / per department / annual subscription | [Vendor commercial] |
| Implementation & configuration | 4 functions (Mgmt Acct, Treasury, Final Reporting, Document Control) | [Vendor commercial] |
| Data migration (Odoo → QUOT PSA) | COA→NCoA mapping, balances, open items, contracts, parallel run | [Vendor commercial] |
| Training | Role‑based, 4 functions + admin | [Vendor commercial] |
| Annual support & maintenance | Typically 18–22% of licence/yr (SLA‑backed) | [Vendor commercial] |
| Hypercare (post‑go‑live) | 4–8 weeks | [Vendor commercial] |

## D. Cost Shape Summary (planning view)

|  | Scenario ① On‑prem + Cloud DR | Scenario ② Cloud‑primary + local backup |
| --- | --- | --- |
| One‑time hardware/facility | ₦18M – 41M (+ NAS) | ₦2M – 5M (NAS + small UPS only) |
| Recurring infrastructure / yr | ₦3M – 7.2M | ₦9M – 22.8M |
| Software/implementation/support | [Vendor commercial] (same either way) | [Vendor commercial] (same either way) |
| 3‑yr infra TCO (excl. licence) | ≈ ₦27M – 63M | ≈ ₦29M – 73M |
| Best when… | OAG has server room + power | Facility/power not yet ready |

Over 3–5 years, Scenario ① typically has the lower total infrastructure cost and higher sovereignty/performance, provided OAG can host a secure, well‑powered server room. Scenario ② trades higher recurring spend for lowest upfront cost and fastest start.

## E. Assumptions

- ~50–200 named users across the four functions; sizing tuned at discovery.

- Hardware import‑priced; FX at ~₦1,550/US$ (volatile — confirm at quote time).

- Existing generator/cooling/rack reduce Scenario ① one‑time cost.

- Licence/implementation/support are vendor commercials, identical across deployment scenarios.

- Prices exclude VAT/duties and any required network upgrades beyond those listed.